Data Communication
Report #1

Student ID: 202438404

Student Name: 유수프존

Assignment

- 1. OSI 7-Layer Model

- Functions and roles of each layer

| - Layer | - Name | - Function |
| --- | --- | --- |
| - 7 | - Application layer | - Closest layer to user and provides protocols that applications use directly for different purposes. For example, opening website requires HTTP and sending email needs SMTP protocols. |
| - 6 | - Presentation layer | - Ensures that data sent between systems is in the right format(representation) by doing data encoding, serialization, en/decryption and compression so that systems understand each other |
| - 5 | - Session layer | - Setting up, maintaining, ending communication sessions. Also, checkpoints helps resuming long file(or any data) transmission instead of restarting in cases of disconnection/failures |
| - 4 | - Transport layer | - Provides end-to-end communication between applications/processes running on different devices. TCP (Transmission Control Protocol) provides reliable and ordered data transmission, while UDP (User Datagram Protocol) is faster but doesn't guarantee delivery or order. |
| - 3 | - Network layer | - Responsible for delivering packets from the source device to the destination across different networks. Main functions are logical addressing (IP) and routing. |
| - 2 | - Data Link layer | - Responsible for delivering frames between devices on the same local network. Main functions are physical addressing (MAC), error detection and controlling access to the shared medium. For example, Ethernet and Wi-Fi work on this layer. |
| - 1 | - Physical layer | - Transmits raw bits (0s and 1s) over the physical medium as electrical signals, light or radio waves. Defines things like cables, connectors, voltages and data rates. |

- Role and functions of headers

- Role of headers is to give information about the details of the data and host (sender or receiver)

- Explanation of modularity

- Modularity in Data Communication is that dividing the data and its headers sent into separate modules so that layers can work with them easily. In each layer, layer specific headers are encapsulated to the data. This makes it modular, easy to check for errors, and maintainable by the corresponding layers of other side.

- Addressing scheme at each layer

7,6,5 Application layer uses domain name or email address; (gachon.ac.kr, yusufjon@gachon.ac.kr)

- Transport layer uses port numbers; (80 for HTTP, 443 for HTTPS)

- Network layer uses IP address; (192.168.0.1)

Data Link layer uses MAC Address (00:1A:2B:3C:4D:5E)

- Physical Layer just transmits electrical signal without using any address scheme.

- Role of the layers on the Internet

- On the internet, only up to 3 layers participate as nodes which can be server, router, or computer etc. For example, the role of router is to move the data from a LAN to another.

- Router uses Physical, Data Link and Network layers. For example, data comes from wires or from signals to the router (Physical) and checks for errors and access using MAC add. (Data Link) and routes data to other LAN networks matching IP address of the receiver.

- 2. Frequency Spectrum.

- The frequency spectrum is divided into named bands. From low to high frequency, they are acoustic, radio, microwave, infrared, visible light, ultraviolet (UV), and X-ray.

- Characteristics of each frequency band:

1)Acoustic: This is the range of sound that humans can hear. No wireless system uses it, but it is used on wired systems such as the telephone and fax.

2)Radio (300 kHz to 300 MHz): Most communication devices use this band, such as AM/FM radio, RFID, and smart keys. For example, a smart key can pass through walls and start a car.

3)Microwave (300 MHz to 30 GHz): This band is used for mobile communication, GPS, satellites, and Wi-Fi. For example, Wi-Fi and microwave ovens use 2.4 GHz, and Wi-Fi also uses 5 GHz.

4)Infrared: It travels in straight lines and is easily reflected. For example, a TV remote control uses infrared.

5)Visible light and UV: These are the highest frequencies used for communication, and they can carry a huge amount of data per unit time. For example, optical fiber uses infrared, visible light, and UV.

6)X-ray: It is harmful to the human body, so no wireless communication devices use frequencies above UV. l In general, a low frequency spreads out widely, passes through obstacles, and carries less information (for example, AM radio). A high frequency travels in a straight line, is reflected by obstacles, and carries more information (for example, FM radio).

- 3. Multiple Access is the method that enables many users share same frequency band at the same time. Sharing is important because if every user cannot use specific frequency channel exclusively. So there are several algorithms to solve this problem:

FDMA - Frequency Division Multiple Access. This algorithm divides bandwidth into smaller pieces  as channels for each user. But still this is very inefficient because the number of users is few

TDMA - Time Division Multiple Access. Users sharing same channel take turns to use time slots. The number of users can be 3 times more than of FDMA, if 3 users share same channel.

CDMA - Code Division Multiple Access. Many users share one block of data at the same time. It is a more unique structure than FDMA or TDMA.

- - Explanation of chip sequences (CDMA)

- Chip sequences are fixed-length sequences of codes assigned to each station. Each station multiplies its data bits by its own chip sequence to spread the data. The encoded signals from multiple CDMA users are then added together and transmitted through the shared channel within the allocated frequency band. At the receiver, the chip sequence of the intended sender is used to extract the original data bit from the combined signal by multiplying the received chip values by the corresponding chip values and summing the products.

- 4. Circuit-Switched Networks

- - Concept: Establishment of dedicated path before data transmission.

- - Operating principle: Connection is established in dedicated path and all data is transmitted in that path until disconnection.

- - Characteristics (or advantages and disadvantages)

- Pros: Dedicated bandwidth provides predictable communication.

- Cons: Low resource utilization and if dedicated path fails -> connection interrupts.

- - Application examples: Early phone networks, PSTN (Phone Switched Telephone Network)

- Packet-Switched Networks

- - Concept: Data is divided into pieces called as packets forwarded by routers on the Internet. Routers route the packets based on the IP addresses.

- - Operating principle: Router route packets independently into different paths

- - Characteristics (or advantages and disadvantages)

- Pros: Efficient resource sharing, statistical multiplexing and packets can be sent in different alternative routes.

- Cons:Routing independently may result in out-of-order delivery and packet loss until it reaches to the receiver.

- - Application examples: Early phone networks, PSTN (Phone Switched Telephone Network)

- Private Networks

- - Concept: Access to the Network is restricted to only authorized users.

- - Operating principle: Access control uses authentication and permissions to prevent unauthorized access. A VPN (Virtual Private Network) creates an encrypted tunnel over a public network to protect data during transmission.

- Pros: Security

- Cons: Costs on infrastructure + administration.

- - Application examples: CCTV and financial networks